# Functional Requirements Summary: Vehicle Infrastructure Integration Data Use Analysis and Processing System

This document provides a consolidated set of functional requirements for the Vehicle Infrastructure Integration Data Use Analysis and Processing system. It merges the useful requirements from the three source documents, removes duplicate or weak requirements, and rewrites the numbering in a clean sequential order.

---

## 1. Core System Data Collection and Processing

### FR-01 VII DUAP Data Collection

**Input:** Probe vehicle data, traffic management system data, weather station data, traveler information system data.

**Output:** Traffic metrics, incident records, road surface assessments, alerts, quality-checked datasets.

**Description:** The system collects real-time data from probe vehicles, traffic management systems, weather stations, and traveler information systems for downstream processing and analysis.

### FR-02 Probe Vehicle Data Processing

**Input:** Vehicle speed, location, heading, direction, brake usage, timestamp.

**Output:** Processed probe vehicle records and derived traffic or road condition inputs.

**Description:** The system processes probe vehicle data for use in traffic metrics, road surface assessments, incident inference, and quality validation.

### FR-03 Traffic Management System Data Processing

**Input:** Signal status, incident reports, road status, infrastructure status, timestamp.

**Output:** Processed traffic management data and alert-ready event inputs.

**Description:** The system ingests and processes data from traffic management systems to support incident inference, traffic status monitoring, and alert generation.

### FR-04 Weather Station Data Processing

**Input:** Temperature, precipitation, humidity, wind speed, visibility, station identifier, timestamp.

**Output:** Processed weather observations and weather-derived inputs for analysis.

**Description:** The system processes real-time weather station observations for traffic analysis, road surface assessment, alerting, and visualization.

### FR-05 Traveler Information Data Processing

**Input:** Traveler advisories, route information, presentation data, external traveler information system records.

**Output:** Traveler-facing information inputs and formatted advisory data.

**Description:** The system ingests traveler information data to support advisories, MI Drive presentation, and dissemination workflows.

### FR-06 Real-Time Traffic Data Storage

**Input:** Validated real-time traffic data, probe vehicle data, traffic management data.

**Output:** Stored traffic data in Oracle 10G database.

**Description:** The system stores real-time traffic data in the Oracle 10G database for analysis, retrieval, visualization, and archival processing.

### FR-07 Data Synchronization Across Sources

**Input:** Traffic data, weather observations, road condition data, timestamps.

**Output:** Synchronized multi-source dataset and synchronization status.

**Description:** The system synchronizes traffic, weather, and road condition data to ensure temporal consistency before analysis, alerting, or visualization.

### FR-08 Infrastructure Data Validation Before Processing

**Input:** Infrastructure update, signal status, road status, signage status, data quality rules.

**Output:** Accepted infrastructure input or rejected input with validation reason.

**Description:** Infrastructure updates must be validated against data quality rules before entering the processing pipeline.

### FR-09 Multi-Source Processing Output Generation

**Input:** Validated traffic, weather, infrastructure, probe vehicle, and traveler information data.

**Output:** Traffic metrics, incident inferences, road surface assessments, quality indicators, alerts.

**Description:** The system integrates validated vehicle and infrastructure data into processed outputs for downstream analytics and presentation.

---

## 2. Traffic Metrics, Incident Inference, and Travel Demand

### FR-10 Traffic Metrics Analysis

**Input:** Real-time traffic data, historical traffic data, weather data, road condition data, analysis parameters.

**Output:** Congestion levels, travel times, queue lengths, traffic alerts.

**Description:** The system analyzes traffic data to derive operational traffic metrics for monitoring, visualization, and alerting.

### FR-11 Historical Traffic Metrics Analysis

**Input:** Archived traffic data, historical time range, analysis parameters.

**Output:** Historical congestion trends, travel time summaries, queue length summaries.

**Description:** The system analyzes historical data to support traffic trend analysis, reporting, and travel demand modeling.

### FR-12 Traffic Metrics Audit Record

**Input:** Analytical run ID, analyst or service identity, input summary, output artifact ID, timestamp.

**Output:** Audit record for the analytical run.

**Description:** Every traffic metrics analysis run must produce an audit record linking inputs, actor identity, run metadata, and output artifacts.

### FR-13 Traffic Incident Inference

**Input:** Traffic metrics, queue lengths, congestion levels, weather data, road surface condition data.

**Output:** Inferred incident record, incident severity, incident duration, updated map display.

**Description:** The system infers potential traffic incidents using processed traffic, weather, and road surface condition data.

### FR-14 Incident Lifecycle Status Propagation

**Input:** Incident state change, incident ID, affected road segment, timestamp.

**Output:** Updated visualization, updated alert publisher state, stakeholder notification.

**Description:** Incident lifecycle status changes must automatically propagate to map views, alert systems, and stakeholder notifications without manual refresh.

### FR-15 Incident State Change Notification

**Input:** Incident ID, previous state, new state, timestamp, affected road segment.

**Output:** Notification to relevant stakeholders.

**Description:** Stakeholders are notified when an incident is created, updated, resolved, or otherwise changes state.

### FR-16 Incident Report Viewing

**Input:** Incident ID, user identity, viewing request.

**Output:** Latest incident details, status history, and viewing audit entry.

**Description:** The system provides incident reports using the latest inferred records and status history, while logging viewing activity.

### FR-17 Incident Detail Recency

**Input:** Incident detail request, incident ID, cache timestamp, current status history.

**Output:** Current incident details or stale-data rejection.

**Description:** Incident reports must use current status history rather than stale cached entries.

### FR-18 Incident Logging Degradation Indicator

**Input:** Incident alert update, logging state.

**Output:** Alert warning flag or degraded logging indicator.

**Description:** Alerts associated with incidents must include warning flags when the underlying incident logging state is degraded.

### FR-19 Travel Demand Tracking

**Input:** Congestion levels, travel times, weather data, historical travel patterns.

**Output:** Travel demand forecast and high-demand alerts.

**Description:** The system forecasts travel demand based on current and historical conditions and generates alerts when forecast demand exceeds capacity thresholds.

### FR-20 Travel Demand Threshold Alert

**Input:** Travel demand forecast, capacity threshold, geographic area.

**Output:** Demand exceedance alert.

**Description:** The system generates alerts when travel demand is expected to exceed predefined infrastructure capacity thresholds.

---

## 3. Road Surface and Weather Condition Processing

### FR-21 Road Surface Condition Calculation

**Input:** Probe vehicle speed, brake usage, weather data, road condition sensor data.

**Output:** Road surface condition assessment, severity, hazardous condition alert, map update.

**Description:** The system calculates road surface condition by combining vehicle-level behavior and environmental data.

### FR-22 Road Surface Assessment Versioning

**Input:** Road surface assessment output, schema version.

**Output:** Version-tagged assessment.

**Description:** Road surface condition assessment outputs must include schema version tags to ensure compatibility with downstream consumers.

### FR-23 Road Surface Recalculation Cascade

**Input:** Updated source dataset, affected road segment, recalculation trigger.

**Output:** Recomputed road surface assessment and recalculation audit record.

**Description:** Updates to source traffic, probe vehicle, or weather datasets must automatically trigger recalculation of dependent road surface assessments.

### FR-24 Road Surface Recalculation Audit

**Input:** Recalculation event, dataset identifier, change timestamp, affected assessment.

**Output:** Recalculation audit log entry.

**Description:** Recalculation events must be logged with source dataset identifiers, timestamps, and affected outputs.

### FR-25 Road Surface Recalculation Failure Recovery

**Input:** Recalculation failure, previous assessment state, affected road segment.

**Output:** Rollback to prior assessment state and failure log.

**Description:** If a road surface recalculation fails, the system restores the prior valid assessment state and records the failure.

### FR-26 Weather Observation Monitoring

**Input:** Weather station observations, probe data, road condition data.

**Output:** Integrated weather and traffic dataset, weather alerts, updated weather displays.

**Description:** The system monitors weather observations and integrates them with traffic and road data for situational awareness.

### FR-27 Weather Visualization Integration

**Input:** Weather-derived insight, weather alert, map overlay target.

**Output:** Updated weather map overlay and display layer.

**Description:** Weather-derived insights and alerts must be propagated to map overlays and display layers without delay.

### FR-28 Current Weather Presentation Consistency

**Input:** Weather display request, latest processed weather data.

**Output:** Display of current weather conditions or currency indicator.

**Description:** Weather presentation methods must ensure users see the most current available weather conditions or an explicit currency indicator.

### FR-29 Weather Fallback for Forecasting

**Input:** Unavailable real-time weather data, recent historical weather data, traveler profile data.

**Output:** Forecast output with fallback indicator or forecast rejection.

**Description:** When real-time weather data is unavailable, the system may use recent historical data within an allowed window and must mark outputs with fallback mode and reason codes.

### FR-30 Forecast Fallback Rejection

**Input:** Forecast request, invalid fallback window, unavailable real-time data.

**Output:** Forecast rejection indicator and recovery recommendation.

**Description:** If weather fallback data is outside the allowed window, the system rejects the forecast and provides recovery guidance.

---

## 4. Alerting and Dissemination

### FR-31 Alert Processing

**Input:** Traffic metrics, weather conditions, road surface assessments, asset conditions, alert thresholds.

**Output:** Generated alert, prioritized alert queue, web interface update.

**Description:** The system processes traffic, weather, road surface, and asset conditions against configured thresholds to generate alerts.

### FR-32 Alert Input Quality Gate

**Input:** Alert candidate data, quality status, synchronization timestamp.

**Output:** Alert evaluation approval or suppressed alert violation log.

**Description:** Alert inputs must be quality-checked and carry a current synchronization timestamp before alert evaluation.

### FR-33 Suppress Alerts from Unvalidated Data

**Input:** Alert candidate generated from unvalidated data.

**Output:** Suppressed alert and data integrity violation log.

**Description:** Alerts based on unvalidated data must be suppressed and logged as data integrity violations.

### FR-34 Traffic Alert Publication

**Input:** Traffic alert, incident data, congestion level, administrator approval.

**Output:** Published traffic alert to configured systems.

**Description:** The system publishes traffic alerts to traffic management and traveler information systems after required approval.

### FR-35 Weather Event Alert Publication

**Input:** Weather event, weather threshold breach, affected area.

**Output:** Published weather alert.

**Description:** The system generates and publishes alerts for weather events such as snow, heavy rain, reduced visibility, or temperature-related hazards.

### FR-36 Asset Condition Alert Publication

**Input:** Asset condition, infrastructure damage, signal malfunction, severity.

**Output:** Published asset condition alert.

**Description:** The system generates and publishes alerts for infrastructure damage, signal malfunction, or other asset condition issues.

### FR-37 Hazardous Alert Dissemination

**Input:** Hazardous condition alert, registered dissemination channels.

**Output:** Alert delivered to MI Drive, TMS, TIS, or other configured channels.

**Description:** Hazardous condition alerts are routed to all registered dissemination channels.

### FR-38 Alert Delivery Archival

**Input:** Disseminated alert, target channel, delivery result.

**Output:** Archived alert delivery record.

**Description:** Every disseminated alert must be archived with delivery status and target channel details.

### FR-39 Alert Delivery Status Tracking

**Input:** Alert delivery attempt, target channel, delivery status.

**Output:** Delivery success or failure status record.

**Description:** The system tracks delivery outcomes for each alert and dissemination channel.

### FR-40 Alert Logging Failure Notification

**Input:** Incident alert update, alert logging failure.

**Output:** Stakeholder notification and degraded logging record.

**Description:** Stakeholders must be notified when incident alert updates cannot be logged.

### FR-41 Unified Traveler Advisory Content

**Input:** Traveler advisory, traffic context, weather context, incident context.

**Output:** Unified notification payload.

**Description:** The system composes notification content that merges traveler advisories with relevant traffic, weather, and incident context.

### FR-42 Presentation-Specific Alert Formatting

**Input:** Alert content, active presentation method, target device or channel.

**Output:** Formatted alert payload.

**Description:** Alerts must be formatted according to the active presentation method and target channel requirements.

---

## 5. Data Quality, Integrity, and Lifecycle Management

### FR-43 Data Quality Checks

**Input:** Incoming traffic, weather, probe vehicle, road condition, or infrastructure data.

**Output:** Validated data, rejected data, quality status, quality log.

**Description:** The system validates incoming data for accuracy, consistency, completeness, timeliness, and schema compliance.

### FR-44 Invalid Data Exclusion

**Input:** Dataset with failed quality checks.

**Output:** Rejected dataset and exclusion from processing pipeline.

**Description:** Invalid data is logged and excluded from downstream processing.

### FR-45 Data Integrity Rule Enforcement

**Input:** Candidate dataset, lifecycle state, integrity rule set.

**Output:** Approved dataset or rejected dataset with integrity violation record.

**Description:** Candidate datasets for analysis must be evaluated against integrity rules based on lifecycle state.

### FR-46 Deterministic Data Rejection Handling

**Input:** Invalid dataset, rejection reason.

**Output:** Rejected dataset record and degraded output indicator where applicable.

**Description:** Invalid data must be recorded as rejected, and dependent outputs must carry degraded indicators when affected.

### FR-47 Quality Rule Reload on Configuration Change

**Input:** Updated quality check configuration.

**Output:** Reloaded rule set applied to subsequent processing.

**Description:** Quality checks must reload and apply new rules immediately after configuration updates.

### FR-48 Data Quality Rule Change Logging

**Input:** Quality rule modification, actor identity, previous value, new value, timestamp.

**Output:** Rule management log entry.

**Description:** Modifications to quality rules are recorded for compliance auditing.

### FR-49 Cache Refresh Cascading to Quality Checks

**Input:** Cache refresh failure, affected cached data.

**Output:** Cache invalidation and quality-check escalation log.

**Description:** Cache refresh failures must trigger cache invalidation and escalation in the quality-check log to prevent stale data from being used in analysis.

### FR-50 Provenance Propagation

**Input:** Processed entity, source data lineage, quality indicator, completeness indicator.

**Output:** Downstream output with inherited provenance and quality metadata.

**Description:** Downstream outputs such as alerts, MI Drive data, and UI displays must inherit source data quality and completeness indicators.

### FR-51 Metadata Enrichment

**Input:** Processed data entity, source identifiers, lineage attributes.

**Output:** Enriched processed entity with fully qualified lineage metadata.

**Description:** The system applies data lineage metadata to processed entities for traceability and compliance.

### FR-52 Data Lifecycle Management

**Input:** Data entity, lifecycle action, actor identity.

**Output:** Created, updated, archived, or deleted entity with lifecycle record.

**Description:** The system manages the create, update, archive, and delete lifecycle for data entities while maintaining integrity and auditability.

### FR-53 Data Lifecycle Failure Recovery

**Input:** Lifecycle operation failure, prior data state.

**Output:** Restored prior state and compensating action log.

**Description:** If a lifecycle operation fails, the system restores the prior state and logs a compensating action.

### FR-54 Lifecycle Transition Audit

**Input:** Entity lifecycle transition, actor, timestamp, previous state, new state.

**Output:** Ordered audit trail entry.

**Description:** The system maintains ordered audit trails for data entity state changes.

### FR-55 Delete Outdated Data

**Input:** Outdated data criteria, data type, time threshold, administrator identity.

**Output:** Deleted outdated records and deletion job log.

**Description:** Authorized administrators can delete outdated data based on configured criteria.

### FR-56 Deletion Job Audit

**Input:** Deletion job ID, criteria applied, affected data range, actor identity.

**Output:** Deletion audit record.

**Description:** Deletion jobs must be logged with job ID, criteria, affected range, actor identity, and timestamp.

---

## 6. Caching, Archiving, and Retrieval

### FR-57 Dynamic Data Caching

**Input:** Traffic metrics, road surface conditions, weather observations, cache policy.

**Output:** Cached data for rapid access and caching operation log.

**Description:** The system caches frequently accessed data to reduce latency and improve web interface performance.

### FR-58 Real-Time Probe Data Cache Validation

**Input:** Probe vehicle data, timeliness window, completeness rule.

**Output:** Cache commit approval or rejection.

**Description:** Real-time probe data must satisfy timeliness and completeness windows before being committed to the cache.

### FR-59 Cache Refresh Failure Rollback

**Input:** Cache refresh operation, refresh failure, prior cache state.

**Output:** Atomic rollback to last consistent cache state.

**Description:** If a cache refresh fails, the system rolls back to the last known consistent cache state.

### FR-60 Stale Data Serving

**Input:** Cache refresh failure, last valid cached data.

**Output:** Served stale data with staleness indicator.

**Description:** When cache refresh fails, the system may serve stale data only with a visible staleness indicator.

### FR-61 Atomic Cache Recovery

**Input:** Cache refresh operation, affected cache keys, failure condition.

**Output:** Restored affected cache entries.

**Description:** The system identifies affected cache keys before refresh so the cache can be fully restored if refresh fails.

### FR-62 Cache Operation Logging

**Input:** Cache population, refresh, eviction, or invalidation event.

**Output:** Cache operation log entry.

**Description:** The system logs cache population, refresh, eviction, and invalidation events.

### FR-63 Cache Consistency Evidence

**Input:** Synchronized access event, resulting cache state.

**Output:** Correlation record linking access event to cache state.

**Description:** The system records evidence linking synchronized access events to resulting cache states.

### FR-64 Cache Corrective Action Indicator

**Input:** Cache change, missing consistency evidence.

**Output:** Corrective action indicator.

**Description:** If consistency evidence is missing after a cache change, the system emits a corrective action indicator.

### FR-65 Long-Term Data Archiving

**Input:** Processed traffic metrics, incident details, road surface assessments, metadata.

**Output:** Archived data in Oracle 10G with traceability metadata.

**Description:** Processed data is archived in Oracle 10G for long-term storage and retrieval.

### FR-66 Archival Failure Continuity

**Input:** Long-term archiving failure, operational log data.

**Output:** Short-term retained logs and continuity degradation indicator.

**Description:** If long-term archiving fails, the system retains logs in short-term operational storage to preserve continuity.

### FR-67 Archival Reconciliation

**Input:** Failed archive item, retry criteria, affected records.

**Output:** Reconciliation report and retry eligibility outcome.

**Description:** Failed archive items are evaluated against retry criteria and reported through reconciliation outputs.

### FR-68 Archival Continuity Notification

**Input:** Archiving continuity degradation, affected data type.

**Output:** Stakeholder notification.

**Description:** Stakeholders are notified when archival or log continuity degrades.

### FR-69 Archived Data Retrieval

**Input:** Authorized role, structured query, data type, time range, outdated data criteria.

**Output:** Archived records or structured retrieval status.

**Description:** Authorized users can query archived data using structured queries and filters.

### FR-70 Explicit Retrieval Status

**Input:** Retrieval request, matching records, filter criteria.

**Output:** Success, not-found, or criteria-mismatch status.

**Description:** Retrieval responses must distinguish between successful retrieval, missing records, and existing records that do not match filters.

### FR-71 Retrieval Access Control

**Input:** Retrieval request, user identity, information type.

**Output:** Retrieval result or access-denied response.

**Description:** Data retrieval must respect access-control rules and must not require consumers to access the database directly.

### FR-72 Alternate Retention Retrieval

**Input:** Incident report view request, archiving failure, alternate short-term storage.

**Output:** Incident report from alternate retention source.

**Description:** If archiving fails, the UI may serve incident reports from alternate short-term retention sources when valid.

---

## 7. User Interface, Visualization, and Geo-Referencing

### FR-73 Web-Based User Interface

**Input:** User request, authorized session, selected information type.

**Output:** Web-based interface view for traffic, weather, road conditions, alerts, or incidents.

**Description:** The system provides a web-based interface for browsing traffic, weather, road condition, incident, and alert information.

### FR-74 Map View Display

**Input:** Real-time metrics, incident records, weather records, road surface assessments, geo-referenced data.

**Output:** Map display with overlays and icon layers.

**Description:** The system renders real-time and processed data overlays on map views.

### FR-75 Michigan Geographic Framework Integration

**Input:** Location data, road segment data, station location, incident location.

**Output:** Geo-referenced output aligned to the Michigan Geographic Framework.

**Description:** All mapped transportation data must be geo-referenced using the Michigan Geographic Framework.

### FR-76 Location Data Association

**Input:** Location data, geo-referencing log, target output.

**Output:** Associated geo-referenced output or explicit association failure status.

**Description:** Location data must be correctly associated with geo-referenced outputs, and failures must be surfaced explicitly.

### FR-77 Geo-Referencing Audit

**Input:** Location association event, geo-referencing log, affected alert or visualization.

**Output:** Geo-referencing audit record.

**Description:** The system audits the association of location data with alerts and visualization outputs.

### FR-78 Icon Layer Management

**Input:** Icon layer identifier, visibility, priority, appearance settings.

**Output:** Updated icon layer configuration.

**Description:** Administrators can manage map icon layer visibility, priority, and appearance.

### FR-79 Display De-Cluttering

**Input:** Map density, user preferences, display settings.

**Output:** Adjusted map display density.

**Description:** The system dynamically adjusts display density to reduce visual overload.

### FR-80 UI Settings Modification

**Input:** Map display option, icon visibility setting, administrator identity.

**Output:** Updated UI setting.

**Description:** Authorized administrators can modify map display options and icon visibility settings.

### FR-81 Presentation Method Configuration

**Input:** Presentation method, display style, administrator configuration.

**Output:** Updated presentation configuration applied to active visualizations.

**Description:** Administrators can configure presentation methods such as 3D maps, heatmaps, and animated traffic flows.

### FR-82 Presentation Method Propagation

**Input:** Updated presentation method selection.

**Output:** Consistent rendering across active visual outputs.

**Description:** Presentation method selections must propagate uniformly across all rendered outputs.

### FR-83 Real-Time Map Rendering Diagnostics

**Input:** Rendering event, map view identifier, rendering log.

**Output:** Diagnostic linkage between rendered view and log.

**Description:** Rendering diagnostics must link specific rendering logs to the real-time map views they produced.

### FR-84 Traffic Status View Synchronization Indicator

**Input:** Traffic status view request, synchronization completeness status.

**Output:** Traffic view with synchronization completeness indicator.

**Description:** Traffic status views must indicate synchronization completeness so users can judge data currency.

### FR-85 Traffic Status Cache TTL Enforcement

**Input:** Traffic status view request, cache timestamp, configured TTL.

**Output:** Current view or stale snapshot rejection.

**Description:** Traffic status map displays must not serve snapshots older than the configured cache time-to-live.

### FR-86 Contextual Traffic Status View

**Input:** Traffic status request, available weather context.

**Output:** Traffic status view with weather context where available.

**Description:** Traffic status views include weather context when valid weather data is available.

### FR-87 Road Condition Viewing

**Input:** Road condition view request, latest quality-checked road assessment.

**Output:** Road condition view.

**Description:** Road condition views must be derived from the latest quality-checked assessments.

### FR-88 Asset Condition Viewing

**Input:** Asset condition view request, latest asset assessment.

**Output:** Asset condition display.

**Description:** Asset condition views must align with the most recent available asset assessments.

---

## 8. Administration, Configuration, and Source Management

### FR-89 Authorized Configuration Administration

**Input:** Administrator identity, configuration change request.

**Output:** Approved configuration update or denied request.

**Description:** Only authorized administrators can modify data use, configuration, processing behavior, and integration settings.

### FR-90 Administrative Task Management

**Input:** Administrative task, task parameters, administrator identity.

**Output:** Completed task record and audit log.

**Description:** Administrators can manage system tasks such as data deletion, configuration updates, and operational maintenance.

### FR-91 Administrative Task Audit Linkage

**Input:** Administrative task log, configuration change record.

**Output:** Linked audit record.

**Description:** Administrative task logs must be integrated with configuration change records for end-to-end auditability.

### FR-92 System Configuration Management

**Input:** Data source setting, alert threshold, caching rule, integration parameter.

**Output:** Updated system configuration.

**Description:** Administrators can maintain system configuration for data sources, thresholds, cache rules, and integration behavior.

### FR-93 Configuration Change Version History

**Input:** Configuration change, prior value, new value, actor, timestamp.

**Output:** Versioned configuration history entry.

**Description:** The system maintains versioned logs of configuration changes.

### FR-94 Configuration Consistency Evaluation

**Input:** Configuration change, current VII and DUAP configuration snapshots.

**Output:** Consistency evaluation result.

**Description:** Configuration changes trigger a consistency evaluation cycle to detect misalignment across components.

### FR-95 VII Configuration Snapshot Alignment

**Input:** VII configuration snapshot, DUAP configuration snapshot.

**Output:** Alignment check result and misalignment indicator.

**Description:** VII configuration snapshots are included in cross-component alignment checks.

### FR-96 Add New Data Source

**Input:** Data source type, connection parameters, data format, source identifier.

**Output:** Registered data source.

**Description:** Authorized administrators can add new data sources such as probe vehicles, weather stations, traffic management systems, and traveler information systems.

### FR-97 Dynamic Source Activation

**Input:** Newly registered source, activation request.

**Output:** Source activated for collection without system restart.

**Description:** New sources can be activated for collection without requiring a system restart.

### FR-98 Source Activation Fallback

**Input:** Source activation failure, previous configuration state.

**Output:** Retained previous state or structured activation failure.

**Description:** If a new source fails to activate, the system determines whether the previous state can be safely retained.

### FR-99 Processing Enablement Block on Configuration Inconsistency

**Input:** Processing enablement request, unresolved configuration consistency state.

**Output:** Processing withheld with reason.

**Description:** The system withholds processing enablement if configuration consistency remains unresolved.

### FR-100 Manage Probe Vehicles

**Input:** Probe vehicle identifier, activation status, communication protocol.

**Output:** Updated probe vehicle configuration.

**Description:** Administrators can manage probe vehicle activation, deactivation, and communication protocols.

### FR-101 Probe Vehicle Lineage

**Input:** Probe vehicle management operation, resulting data state.

**Output:** Linked operation log and data state record.

**Description:** Probe vehicle management operation logs must link directly to the resulting data state.

### FR-102 Manage Traffic Management System Integration

**Input:** TMS endpoint, communication format, integration setting.

**Output:** Updated TMS integration configuration.

**Description:** Administrators can configure traffic management system endpoints and formats.

### FR-103 Manage Weather Stations

**Input:** Weather station identifier, communication protocol, station configuration.

**Output:** Updated weather station configuration.

**Description:** Administrators can configure weather stations and their communication protocols.

### FR-104 Weather Station Administration Authorization

**Input:** User role, weather station operation, station identifier.

**Output:** Authorized operation or rejected attempt with audit log.

**Description:** CRUD operations on weather station records require role-based authorization.

### FR-105 Manage Traveler Information Systems

**Input:** Traveler information endpoint, data format, delivery setting.

**Output:** Updated traveler information system configuration.

**Description:** Administrators can manage delivery endpoints and formats for traveler information systems.

### FR-106 Manage DUAP Modules

**Input:** DUAP module identifier, module setting, processing rule.

**Output:** Updated DUAP module configuration.

**Description:** Administrators can manage DUAP modules, algorithms, and processing rules.

### FR-107 Manage VII System

**Input:** VII integration rule, data source detail, system configuration.

**Output:** Updated VII system configuration.

**Description:** Administrators can manage VII system integration rules and data source details.

---

## 9. Algorithm, Rule, and Format Management

### FR-108 Manage Data Processing Algorithm

**Input:** Algorithm identifier, algorithm parameters, action type.

**Output:** Added, updated, deleted, or retrieved algorithm.

**Description:** Administrators can add, update, delete, or retrieve processing algorithms.

### FR-109 Mandatory Algorithm Test Run

**Input:** Algorithm update, historical test dataset.

**Output:** Test run result and approval or rejection.

**Description:** Algorithm additions or replacements must include mandatory test runs using historical data before activation.

### FR-110 Algorithm Activation Failure Rollback

**Input:** Algorithm activation failure, prior active algorithm.

**Output:** Restored prior algorithm and failure log.

**Description:** Failed algorithm activation must roll back to the previous active algorithm state.

### FR-111 Algorithm and Processing Rule Versioning

**Input:** Algorithm or processing rule update, version metadata.

**Output:** Versioned algorithm or rule record.

**Description:** The system tracks versions of algorithms and processing rules used by DUAP modules.

### FR-112 Manage Data Quality Checks

**Input:** Validation criteria, threshold values, action type.

**Output:** Updated data quality check configuration.

**Description:** Administrators can define and modify validation criteria and threshold values.

### FR-113 Manage Alert Configuration

**Input:** Alert threshold, severity rule, routing rule, administrator identity.

**Output:** Updated alert configuration.

**Description:** Administrators can configure alert thresholds, severity mappings, and routing rules.

### FR-114 Alert Configuration Change Logging

**Input:** Alert configuration modification, actor identity, previous value, new value.

**Output:** Rule management log entry.

**Description:** Modifications to alert configurations must be recorded in Rule Management Logs.

### FR-115 Output Format Management

**Input:** Output format definition, target format, mapping rules.

**Output:** Updated output format configuration.

**Description:** Administrators can modify output formats such as CSV, JSON, XML, SAE J2354, and TMDD.

### FR-116 Data Format Propagation

**Input:** Updated formatting rule, export channel set.

**Output:** Formatting rule applied to all export channels.

**Description:** Formatting rules managed in settings must propagate consistently across all export channels.

### FR-117 Output Format Compliance Re-Evaluation

**Input:** Output format change, applicable standard.

**Output:** Standards compliance check result.

**Description:** Any change to output formats triggers a re-evaluation of standards compliance.

### FR-118 Formatting Traceability

**Input:** Formatting change event, standards processing run.

**Output:** Linked traceability record.

**Description:** Formatting change events must be linked to the standards processing runs they influence.

### FR-119 SAE J2354 and TMDD Formatting

**Input:** Processed traffic metrics, weather observations, incident details, road condition data.

**Output:** Standardized data for SAE J2354 and TMDD interfaces.

**Description:** The system formats processed data to meet SAE J2354 and TMDD standards.

### FR-120 MDIT Compliance

**Input:** Processed data, interoperability standard requirements.

**Output:** Compliance-approved data or compliance failure.

**Description:** Processed data must comply with Michigan Department of Information Technology standards.

### FR-121 MI Drive Presentation Data Support

**Input:** Traffic data, road condition data, alert data, traveler information data.

**Output:** Validated and formatted MI Drive presentation data.

**Description:** The system generates and validates data for MI Drive presentation.

---

## 10. External System Integration and Dissemination

### FR-122 Update Traffic Management Systems

**Input:** Latest traffic data, road condition data, incident alert data.

**Output:** Updated external traffic management system records.

**Description:** The system synchronizes traffic data and road condition updates with external traffic management systems.

### FR-123 Retrieve Traveler Information

**Input:** Travel time, congestion level, route suggestion, traveler information request.

**Output:** Formatted traveler information.

**Description:** The system retrieves and formats travel times, congestion levels, and route suggestions for traveler dissemination.

### FR-124 External Traveler Advisory Retrieval

**Input:** Traveler advisory retrieval criteria, user identity.

**Output:** Traveler advisory record or structured retrieval status.

**Description:** The system supports structured retrieval of traveler advisories with explicit success, not-found, or criteria-mismatch outcomes.

### FR-125 External Channel Delivery Endpoint Management

**Input:** Channel identifier, endpoint URL, protocol, format.

**Output:** Updated delivery endpoint configuration.

**Description:** Administrators can manage delivery endpoints for external channels such as MI Drive, TMS, and TIS.

### FR-126 External Integration Access Security

**Input:** External integration request, access-control policy.

**Output:** Approved integration access or denied request.

**Description:** External integrations and retrieval operations must respect system access-control rules.

---

## 11. Security and Access Control

### FR-127 Authorized Information Viewing

**Input:** User identity, requested information type, access policy.

**Output:** Information view or access-denied response.

**Description:** Viewing of incidents, traveler information, and sensitive system data is restricted to authorized users.

### FR-128 Access Denial Reason Logging

**Input:** Denied access request, user identity, information type, denial reason.

**Output:** Audit log entry with denial reason.

**Description:** Denied viewing or administrative requests must include a reason in audit logs.

### FR-129 Role-Based Administrative Authorization

**Input:** User role, administrative operation, target resource.

**Output:** Authorized operation or rejected attempt.

**Description:** Administrative operations require role-based authorization.

### FR-130 Configuration Administration Authorization

**Input:** Administrator identity, configuration modification request.

**Output:** Authorized configuration update or denied request.

**Description:** Only authorized administrators can modify data use, processing behavior, sources, thresholds, and system settings.

### FR-131 Secure Database Access Through JDBC

**Input:** Database operation request, JDBC connection credentials, SQL statement.

**Output:** Executed database transaction or database access error.

**Description:** The system uses JDBC for secure and efficient Oracle 10G database access.

### FR-132 Oracle Database Management

**Input:** Insert, update, delete, or query operation; administrator identity.

**Output:** Database operation result and audit record.

**Description:** Authorized administrators can manage Oracle 10G database records while maintaining data integrity.

### FR-133 Standard SQL Query Execution

**Input:** Authorized SQL query, query parameters.

**Output:** Query result for reporting or analysis.

**Description:** Authorized administrators can execute standard SQL queries for reporting and analysis.

---

## 12. Processing Resilience and Deferred Execution

### FR-134 Processing Environment Unavailability Fallback

**Input:** Analysis request, unavailable processing environment, original parameters.

**Output:** Deferred analysis queue entry with idempotency key.

**Description:** If the processing environment is unavailable, analysis jobs are enqueued with original parameters and idempotency keys.

### FR-135 Deferred Analysis Resumption

**Input:** Deferred analysis queue entry, recovered processing environment.

**Output:** Resumed analysis job without duplicate processing.

**Description:** Deferred analyses resume after environment recovery using idempotency keys to prevent duplicate execution.

### FR-136 Deferred Analysis Terminal Failure

**Input:** Deferred analysis job, expiration threshold.

**Output:** Permanently failed analysis status and user-facing failure record.

**Description:** Deferred analyses that exceed expiration thresholds are marked as permanently failed and surfaced to users.

### FR-137 Idempotent Processing Recovery

**Input:** Retried operation, idempotency key, prior execution state.

**Output:** Stable outcome without duplicate processing.

**Description:** The system uses idempotency keys to support retry-safe processing recovery.

### FR-138 Processing Fallback User Status

**Input:** Deferred or failed processing job, user-facing status request.

**Output:** Current processing status and failure reason when applicable.

**Description:** Users can see whether analysis is pending, resumed, completed, or permanently failed.

### FR-139 Terminal Outcome Recording

**Input:** Processing job, terminal outcome, timestamp.

**Output:** Recorded terminal outcome.

**Description:** The system records terminal outcomes for deferred, completed, or failed analysis jobs.

---

## 13. Audit, Logging, and Compliance Traceability

### FR-140 Administrative Audit Logging

**Input:** Administrative action, actor identity, target resource, timestamp, change summary.

**Output:** Tamper-evident administrative audit record.

**Description:** The system records tamper-evident logs for administrative tasks and configuration changes.

### FR-141 System Change Log Linkage

**Input:** System change log, administrative audit trail record.

**Output:** Linked evidence record.

**Description:** System change logs must be linked directly to administrative audit trails.

### FR-142 Rule Management Logging

**Input:** Rule modification, actor identity, timestamp, affected rule.

**Output:** Rule management log entry.

**Description:** Changes to alert configurations and quality check rules are logged for compliance.

### FR-143 Combined Audit Evidence

**Input:** Administrative audit record, system change log, configuration change record.

**Output:** Combined evidence trail.

**Description:** The system links administrative, configuration, and system change logs for end-to-end traceability.

### FR-144 Compliance Check Correlation

**Input:** Compliance check log, quality status indicator.

**Output:** Correlated compliance evidence.

**Description:** Compliance check logs must be correlated to quality status indicators.

### FR-145 Analytical and Deletion Audit

**Input:** Analytical run, deletion job, actor, criteria, affected data range.

**Output:** Audit record covering analysis or deletion.

**Description:** Analytical runs and deletion jobs are logged for complete traceability.

### FR-146 Viewing Activity Audit

**Input:** User identity, viewed record type, record identifier, timestamp.

**Output:** Viewing activity audit entry.

**Description:** Viewing activity for sensitive information such as incident reports is logged.

### FR-147 Audit Log Version History

**Input:** Configuration change, lifecycle transition, actor, prior value, new value.

**Output:** Versioned audit history.

**Description:** The system maintains versioned history for configuration and lifecycle changes.

### FR-148 Audit Continuity Degradation Notification

**Input:** Audit or log continuity degradation event.

**Output:** Stakeholder notification.

**Description:** Stakeholders are notified when audit or log continuity is degraded.

---

## 14. Technical Infrastructure

### FR-149 Java Software Foundation

**Input:** DUAP processing components and application modules.

**Output:** Java-based processing and storage operations.

**Description:** The system uses a Java software foundation for core processing and application logic.

### FR-150 Oracle 10G Database Support

**Input:** Traffic data, weather data, incident records, road surface assessments, audit logs.

**Output:** Persisted records in Oracle 10G.

**Description:** Oracle 10G serves as the primary database for system data, processed records, archives, and logs.

### FR-151 JDBC Database Access

**Input:** Java database operation, SQL command, connection configuration.

**Output:** Executed database operation through JDBC.

**Description:** JDBC is used for secure and efficient access to the Oracle 10G database.

### FR-152 Standards-Compliant Data Processing

**Input:** Processed data, MDIT, SAE J2354, and TMDD requirements.

**Output:** Standards-compliant data outputs.

**Description:** The system processes and formats data according to applicable MDIT, SAE J2354, and TMDD standards.

### FR-153 Supported Output Formats

**Input:** Processed data, selected output format.

**Output:** CSV, JSON, XML, SAE J2354, or TMDD formatted output.

**Description:** The system supports multiple output formats for internal and external use.

---

## 15. General Reliability and User-Facing Outcomes

### FR-154 Structured Failure Response

**Input:** Failed operation, failure reason, affected entity.

**Output:** Structured failure response.

**Description:** The system returns structured failure responses for failed operations and exposes relevant failure reasons to users or operators.

### FR-155 Degraded Output Indicator

**Input:** Output generated from incomplete, stale, fallback, or degraded source data.

**Output:** Output with degraded indicator and reason code.

**Description:** Outputs affected by degraded data must be marked with a degraded indicator and reason code.

### FR-156 Synchronization Completeness Indicator

**Input:** View request, synchronized data status.

**Output:** Displayed synchronization completeness status.

**Description:** Views based on synchronized data must display synchronization completeness when relevant.

### FR-157 User-Facing Processing Status

**Input:** Analysis job, lifecycle operation, retrieval request.

**Output:** Clear user-facing status.

**Description:** The system provides clear statuses such as pending, successful, failed, not found, criteria mismatch, degraded, or permanently failed.

### FR-158 Staleness Indicator

**Input:** Served stale data, cache age, data source status.

**Output:** Visible staleness indicator.

**Description:** Stale data must be visibly marked so users can understand its currency.

### FR-159 Explicit Recovery Recommendation

**Input:** Recoverable failure, rejected forecast, failed processing operation.

**Output:** Recovery recommendation for operator or user.

**Description:** Where practical, the system provides recovery recommendations for failed or rejected operations.

### FR-160 Consistency Blocking for Unsafe Processing

**Input:** Processing request, unresolved consistency state, invalid configuration, invalid data state.

**Output:** Processing blocked with reason.

**Description:** The system blocks unsafe processing when consistency or data validity cannot be established.